XYZ Company

Total Number of Hosts: 600

Departments		HB (2n-2)	NB (32-HB)	CIDR	Subnet Mask

Sales		400	9		23		/23	255.255.254.0

Marketing	180	8		24		/24	255.255.255.0

Admin		12	4		28		/28	255.255.255.240

IT		8	4		28		/28	255.255.255.240

Find the following for each subnet:

- Subnet Address

- Broadcast Address

- Host IP Address Range

- Subnet Mask

Find the Host IP address of the following subnets:

- IP Address of Host # 250, 255, 256, 306, 400 of Sales Department

- IP Address of Host # 128, 180 of Marketing Department

Solution

Range

Class A	0.0.0.0		127.255.255.255

Class B	128.0.0.0	191.255.255.255

Class C	192.0.0.0	223.255.255.255

Private IP Address Block

Class A	10.0.0.0	10.255.255.255

Class B	172.16.0.0	172.31.255.255

Class C	192.168.0.0	192.168.255.255

NB	HB	CIDR

Class A		8	24	/8	Total 28 Network where every network having 224 hosts

Class B		16	16	/16	Total 216 Network where every network having 216 hosts

Class C		24	8	/24	Total 224 Network where every network having 28 hosts

Network Baskets

| Class A 28 | Class B 216 | Class C 224 |
| --- | --- | --- |

Which Network Class to Choose? – The decision is based on Total Number of Hosts

Network 	172.16.0.0/16

Rule: Any unused subnet can be further subnetted.

Sales need a /23 subnet

/16  /23

172.16.	  0000000   0.00000000

0000001

0000010

0000011

……………

1111111

We get a total Number of /23 subnet = 27 = 128

Assigning the 1st subnet to Sales Department

172.16.	0000000 0.00000000/23

Marketing needs a /24 subnet

Subnetting further the 2nd /23 subnet

/23  /24

172.16.	 0000001   0.   00000000

1

Admin needs a /28 subnet

IT needs a /28 subnet

/23  /28

172.16.	 0000010   0.0000   0000

0.0001

0.0010

…………

1.1111

We get a total Number of /28 subnet = 25 = 32

Assigning the 1st /28 subnet to Admin Department

172.16.	 0000010   0.0000   0000/28

Assigning the 2nd /28 subnet to IT Department

172.16.	 0000010   0.0001   0000/28

Sales

172.16.	0000000 0.00000000/23	Network Address	172.16.0.0/23

0.00000001		# 1 Host IP		172.16.0.1

0.11111010		# 250 Host IP		172.16.0.250

0.11111111		# 255 Host IP		172.16.0.255

1.00000000		# 256 Host IP		172.16.1.0

1.00110010		# 306 Host IP		172.16.1.50

1.10010000		# 400 Host IP		172.16.1.144

1.11111111		Broadcast Address	172.16.1.255

Host IP Address Range: 172.16.0.1 – 172.16.1.254

Marketing

172.16.	 0000001   0.   00000000/24	Network Address	172.16.2.0/24

# 128 Host IP		172.16.2.128

# 180 Host IP		172.16.2.180

11111111		Broadcast Address	172.16.2.255

Host IP Address Range: 172.16.2.1 – 172.16.2.254

Admin

172.16.	 0000010   0.0000   0000/28	Network Address	172.16.4.0/28

1111	Broadcast Address	172.16.4.15

Host IP Address Range: 172.16.4.1 – 172.16.4.14

IT

172.16.	 0000010   0.0001   0000/28	Network Address	172.16.4.16/28

172.16.	 0000010   0.0001   1111	Broadcast Address	172.16.4.31

Host IP Address Range: 172.16.4.17 – 172.16.4.30